# Using the DTMF Controller for the VK5DJ Repeater Controller

Remote controls:

| Function Hex | Value Hex | Description | Default |
| --- | --- | --- | --- |
| 00 | 00 | enable both TXs and timeout timer | 00 |
|  | 01 | inhibit main TX and link TX | 00 |
|  | 02 | inhibit link TX | 00 |
|  | 03 | inhibit timeout | 00 |
|  | 04 | pulse the secondary port for ½ sec | 00 |
| 01 | 00 | Second port behaves as Link | 00 |
|  | 01 | Second port behaves as Gateway | 00 |
|  | 02 | 3sec delay callsign mode | 00 |
|  | 04 | Swedish callsign mode | 00 |
|  | 06 | Italian callsign mode | 00 |
|  | 08 | Use 1750 beep | 00 |
|  | 10 | Use extended tail | 00 |
|  | 20 | Enable secondary port 0=off, 20=on | 00 |
|  | 40 | No callsign | 00 |
| 02 | 00 | no DTMF relay, no beeps | 00 |
|  | 01 | DTMF relayed on main TX | 00 |
|  | 02 | DTMF relayed on link TX | 00 |
|  | 04 | No Roger beeps on tails if set | 00 |
|  | 08 | DTMF provided by external receiver – no mute | 00 |
| 03 | 00 | No CTCSS required for full operation | 00 |
|  | 01 | Require CTCSS for gate/link to TX | 00 |
|  | 02 | Require CTCSS on rptr rcvr for any to TX | 00 |
|  | 04 | Require CTCSS on rptr rx to operate gateway TX | 00 |
|  | 08 | Extended timeout to 1 hour with CTCSS | 00 |
|  | 10 | CTCSS extended operation mode | 00 |
|  | 20 | CTCSS or COS mute operation | 00 |
| 04 | 00-4B | CTCSS delay in minutes after firing | 0A |
| 05 | 00-FF | Mute delay in ms | 00 |
| 06 | 00-4B | Callsign delay in mins | 0A |
| 07 | 00-4B | Timeout in mins | 03 |
| 08 | 00-96 | Primary tail in 1/10 secs | 0A |
| 09 | 00-4B | Message delay in mins | 1D |
| 0A | 00-13 | Postbeep and mode timer (secs) | 0F |
| 0B | 00-19 | Morse speed see comments | 04 |
| 0C | string + FF | Change callsign |  |
| 0D | string + FF | Change msg content |  |
| 0E | 00 | 0E=0* to clear permissions set by function 0 |  |
| 10 | 05-7F | Freq for morse – values from 05 to 7F | $76 |

NOTE: Functions 01-03 REQUIRE one entry of ALL needed options. For example in Function 01 you send 61 if you want Gateway mode On and no callsign (01+20+40).

# Using the DTMF controller

The keypad has command functions and is also used to enter numeric data.

Command functions

The hexadecimal menu order is:

The following may be changed using the edit function “0” on the keyboard

Password (You must enter 4 digits)

Commands “00” to “0B” (You must enter 2 digits)

Callsign entry  (You may enter up to 96 digits equating to 48 Morse characters)

Message entry (You may enter up to 96 digits equating to 48 Morse characters)

Commands “0F” to “10” (note that 0F is actually unused in the repeater controller)

Next follow inbuilt commands that use the stored password.

Timer OFF

Timer ON

Repeater OFF

Repeater ON

CTCSS ON

CTCSS OFF

Set call timer to 10 minutes

Set timeout timer to 3 minutes

Lastly is a menu item to change the frequency of the CTCSS tone. The current stored tone is shown on the first line and changes must always be entered in 4 digits. To enter 67Hz the numerals “0067” should be entered with the keyboard i.e. leading zeroes to pad out to a 4 digit number.

The active keys on the keyboard in menu mode are:

“*” moves to previous menu including a wrap around from Password to CTCSS tone frequency.

“#” moves to next menu including a wrap around from CTCSS tone frequency to Password.

“0” changes from menu mode to edit mode

“D” sends the DTMF codes for the command selected using the stored password.

In Edit Mode the keys of the keyboard input the expected HEX numbers, note that “*” inputs an “E” while “#” inputs an “F”

So for the VK5DJ Repeater Controller the password will always begin with an “E”. A typical password will look like “E973”

# Push buttons

There are three push buttons and one SPST switch

“Save” button is connected to pin 15 of the PIC. Press this when you wish to save the edit data. In all but the “Callsign” and “Message” modes the word “Save” appears when you reach the required number of digits, but as the size of a callsign or message is indeterminate you may press this button when you have completed the input required. Special note: you do not need to enter the “FF” to complete the DTMF sequence – this is automatically appended.

BackSpace button is connected to pin 16 of the PIC.  It works when in edit mode and destructively backspaces during the entry of digits.

Reset button connected to pin 17 of the PIC works by abandoning the current edit process or menu item and returning the machine to the opening “Password” menu item.

# CTCSS Operation

When the CTCSS switch on pin 18 of the PIC is thrown, the PIC moves into a CTCSS frequency mode as determined by the frequency entered in the last menu position.

Because it is necessary to monitor the switch port from time to time, the CTCSS is actually sent as a series of 2 second bursts. These bursts are very close together and should not impact on the decoding of CTCSS in the controller.

John Drew (VK5DJ)

12 October 2009