# Specifications for Port Expander for the VK5DJ controller

- Provide 4 digital inputs (possible uses include burglar alarm, AC fail, EOM flag for speech audio, 1 spare)

- Provide 2 analogue to digital converters (possible uses battery voltage, SWR)

- Provide 5 outputs where possible uses include audio board control  (record, playback, 2 for message number require 4 outputs and 1 of the above inputs to provide TX control for EOM, antenna selection, tone output, switch for siren on breakin.

PIC connections for 16F1827 (required to make A/D conversion available)

| Pin number | Function | Notes |
| --- | --- | --- |
| 1 RA2 | Digital output | TTL drive auxiliary uncommitted |
| 2 RA3 | Digital input | 10K to +5V (possible transistor driver from door SW) |
| 3 RA4 | Digital input | 10K to +5V (possible transistor driver from AC fail) |
| 4 RA5 | Digital input | 10K to +5V (possible transistor driver spare) |
| 5 Vss | Ground power | Ground |
| 6 RB0 | Digital input (interrupt on change to pick up short pulse) | 10K to +5V and to EOM on audio board |
| 7 RB1 | USART input | RX RS232 – TTL (to pin 16 of main PIC on controller) |
| 8 RB2 | USART output | TX RS232 – TTL (to pin 4 of main PIC on controller) |
| 9 RB3 | Digital output | TTL drive to audio board -record |
| 10 RB4 | Digital output | TTL drive to audio board -playback |
| 11 RB5 | Digital output | TTL drive to audio board – message select |
| 12 RB6 | Digital output | TTL drive to audio board – message select |
| 13 RB7 | Digital output | TTL drive auxiliary – siren or antenna |
| 14 Vdd | +5V supply | +5V regulated and bypassed |
| 15 Osc1 | 10 MHz crystal bypassed with 27pf to ground | 10 MHz Crystal for comms stability (TTL RS232) across pins 15 and 16 |
| 16 Osc2 | 10 MHz crystal bypassed with 27pf to ground |  |
| 17 RA0 | A/D conversion 0 | Possible battery volts |
| 18 RA1 | A/D conversion 1 | Possible SWR |

It will be necessary to protect the inputs with long leads. This would apply to the burglar alarm and possibly the AC Fail circuitry. I will use a 0V on an input pin to indicate an active function. All inputs should have a 10k resistor to +5V.  Protections on inputs should be considered, e.g.  a simple transistor switch or alternatively zener diodes or reversed diodes.

Outputs to audio board can be direct connections as this is all TTL. Any outputs to relays will need to be buffered by transistors, for outputs I have made it active high on the pin of the PIC.

A/D input will need to scale the voltage to within a range 0-5V or part thereof.

John Drew VK5DJ						Third draft 4 March 2013

## Coding from controller

Initial response from external expansion is either Acknowledge ( 11), or  Error (00)

In the case of commands 00, 99, AA the result following the Ack is two digits containing the value.

For commands   11,22,33,44 – the playback functions, after the initial Ack there will be a response of an additional Ack when the carrier should drop.

For commands  55,66,77,88 – the record functions - once the record is begun the repeater controller will send a command of FF when the carrier drops, this will turn off the record function.

| Command main | Function | Response remote | Function |
| --- | --- | --- | --- |
| 00 | Request inputs | 11 Ack then byte/byte of input ports 00000xxx | TBA |
| 11 playback msg 1 | Initiate msg 1 | 11 Ack then further Ack to switch off carrier | Play msg 1 |
| 22 playback msg 2 | Initiate msg 2 | 11 Ack then further Ack to switch off carrier | Play msg 2 |
| 33 playback msg 3 | Initiate msg 3 | 11 Ack then further Ack to switch off carrier | Play msg 3 |
| 44 playback msg 4 | Initiate msg 4 | 11 Ack then further Ack to switch off carrier | Play msg 4 |
| 55 record msg 1 | record msg 1 | 11 Ack | record msg 1 |
| 66 record msg 2 | record msg 2 | 11 Ack | record msg 2 |
| 77 record msg 3 | record msg 3 | 11 Ack | record msg 3 |
| 88 record msg 4 | record msg 4 | 11 Ack | record msg 4 |
| 99 report A/D on 0 | Get A/D 0 voltage | Ack then 00-50 | 00 to 50 representing 10.0 to 15.0V which will be sent in morse by main controller |
| AA report A/D on 1 | Get A/D 1 voltage | Ack then 00-FF | TBA |
| BB output 1 off RA5 | Turns off output 1 | 11 acknowledge | TBA |
| CC output 1 on RA5 | Turns on output 1 | 11 acknowledge | TBA |
| DD output 2 off RB7 | Turns off output 2 | 11 acknowledge | TBA |
| EE output 2 on RB7 | Turns on output 2 | 11 acknowledge | TBA |
| FF cancel record | Turns off portB.3 | 11 acknowledge | Cancel record mode |

Command byte is duplicated to create simple error detection. If bytes differ then an error message of 00 is returned.

If an audio recorder supports only one message use playback/record 1.