Operating manual for the DTMF sender

# Purpose

The device has been built to assist repeater control stations manage their local repeater. By storing password and providing many of the common commands as stored messages, key strokes and more importantly errors, are reduced.

There are three user controlled memories. One holds the password and is not disturbed by any of the other entries. The second is used to store the commands – this is a single memory which may contain any of the normal command parameters, including callsign or message. If you enter and send data for say command 01 then you need to separately enter the data for command 02 as the memory is shared. The third memory location stores the frequency of the CTCSS so this is not disturbed by other entries.

# The four buttons are earthed to activate, they are:

# Send CTCSS = PORTC.3 (pin 18)

Closing this switch will, at any time, send the CTCSS signal as set in the menu.

# CursorLeft = PORTC.1 (pin 16)

Pressing this button moves the cursor back one position (destructive backspace) when in edit mode.

# ResetSW = PORTC.2 (pin 17)

This button resets the PIC to start. That is displaying the “Password” screen. It will, if pressed in edit mode abandon the current operation.

# SaveData = PORTC.0 (pin 15)

When in data entry mode pressing this button will save the data. Except for the “Callsign entry” and “Message entry” modes, the save button will work only when the “Save?” invitation is offered.

With “Callsign” and “Message Entry” as the length of the string is indeterminate, the “Save” button may be pressed at any time. The “FF” expected at the end of string for these latter two modes is automatically appended. E.g. with this device a Callsign of VK5DJ would be entered as :

56 4B 35 44 4A

Whereas if a normal DTMF pad was being used a string of:

56 4B 35 44 4A FF FF would be sent

# Keypad operation

When in command mode the following keys operate as function keys:

Star (*) decreases the menu item and rolls over from “Password entry” to the last item.

CrossHatch (#) increases the menu items and rolls over from the last item to “Password entry”.

Zero (0) sets edit mode from which point the keyboard keys input the values expected viz 0,1,2,3,4,5,6,7,8,9,A,B,C,D,E,F (where *=E and #=F). Note that all entries to the controller are in HEX.

The “Save” button saves the edit, or “Reset” abandons the entry and returns to position 1.

# Commands:

These follow the items in the repeater controller manual

| Function Hex | Value Hex | Description | Default |
| --- | --- | --- | --- |
| 00 | 00 | enable both TXs and timeout timer | 00 |
|  | 01 | inhibit main TX and link TX | 00 |
|  | 02 | inhibit link TX | 00 |
|  | 03 | inhibit timeout | 00 |
|  | 04 | pulse the secondary port for ½ sec | 00 |
| 01 | 00 | Second port behaves as Link | 00 |
|  | 01 | Second port behaves as Gateway | 00 |
|  | 02 | 3sec delay callsign mode | 00 |
|  | 04 | Swedish callsign mode | 00 |
|  | 06 | Italian callsign mode | 00 |
|  | 08 | Use 1750 beep | 00 |
|  | 10 | Use extended tail | 00 |
|  | 20 | Enable secondary port 0=off, 20=on | 00 |
|  | 40 | No callsign | 00 |
| 02 | 00 | no DTMF relay, no beeps | 00 |
|  | 01 | DTMF relayed on main TX | 00 |
|  | 02 | DTMF relayed on link TX | 00 |
|  | 04 | No Roger beeps on tails if set | 00 |
|  | 08 | DTMF provided by external receiver – no mute | 00 |
| 03 | 00 | No CTCSS required for full operation | 00 |
|  | 01 | Require CTCSS for gate/link to TX | 00 |
|  | 02 | Require CTCSS on rptr rcvr for any to TX | 00 |
|  | 04 | Require CTCSS on rptr rx to operate gateway TX | 00 |
|  | 08 | Extended timeout to 1 hour with CTCSS | 00 |
|  | 10 | CTCSS extended operation mode | 00 |
|  | 20 | CTCSS or COS mute operation | 00 |
| 04 | 00-4B | CTCSS delay in minutes after firing | 0A |
| 05 | 00-FF | Mute delay in ms | 00 |
| 06 | 00-4B | Callsign delay in mins | 0A |
| 07 | 00-4B | Timeout in mins | 03 |
| 08 | 00-96 | Primary tail in 1/10 secs | 0A |
| 09 | 00-4B | Message delay in mins | 1D |
| 0A | 00-13 | Postbeep and mode timer (secs) | 0F |
| 0B | 00-19 | Morse speed see comments | 04 |
| 0C | string + FF | Change callsign |  |
| 0D | string + FF | Change msg content |  |
| 0E | 00 | 0E=0* to clear permissions set by function 0 |  |
| 10 | 05-7F | Freq for morse – values from 05 to 7F | $76 |

NOTE: Functions 01-03 REQUIRE one entry of ALL needed options. For example in Function 01 you send 61 if you want Gateway mode On and no callsign (01+20+40).

A normal command sequence using this remote controller device would be something like this:

- Turn on device

- Set the correct password if necessary (may not need setting if the displayed password is correct in which case skip this step) by pressing “0” and then entering the password. Press “Save” when offered.

- To send a command – for example “inhibit timeout” press the # key on the keyboard to access the next menu item which is “Command=00”, press “0” to edit, enter “03” and press save.

- Press “D” on the keyboard. You will see the password, command 00 and the parameter 03 being sent. DTMF tone appears on PortE.2 of the PIC (pin 10).

- If you want to send it again just press “D” on the keyboard.

Sending a canned message:

The device has a canned message facility for certain frequently used commands. These are:

Timer OFF, Timer ON, Repeater OFF, Repeater ON, CTCSS ON, CTCSS OFF, Callsign timer 10min, Timer (timeout) 3 mins. Once the password has been entered and stored to send any of the canned messages it is only necessary to move to the correct menu item and press “D”.

To change the CTCSS frequency move to the correct menu item: “Freq =”, press “0” to edit and type in the frequency in HZ. Fractional Hertz may not be entered but fractions are unnecessary due to the broad nature of the CTCSS decoder in the controller board.

# Output pins:

DTMF sound out appears on PortE.2 (pin 10)

CTCSS sound out appears on PortD.1 (pin 20)

PTT line is on PortA.5 (pin 7)

LED on PortD.0 (pin 19)

Both these go high one second before the DTMF is sent, then immediately low after the DTMF is finished.

Questions and comments to John VK5DJ

jdrew@seol.net.au								8 November 2009